NOTIFICATION

The following notification is being circulated in accordance with Article 10.6

| 1. | Notifying Member: EGYPT If applicable, name of local government involved (Article 3.2 and 7.2): |
| --- | --- |
| 2. | Agency responsible: Egyptian Organization for Standardization and Quality 16 Tadreeb El-Modarrebeen St., Ameriya, Cairo - Egypt E-mail: eos@idsc.net.eg/eos.tbt@eos.org.eg Website: http://www.eos.org.eg Tel.: 0222845528 Fax: 0222845504 Name and address (including telephone and fax numbers, email and website addresses, if available) of agency or authority designated to handle comments regarding the notification shall be indicated if different from above: |
| 3. | Notified under Article 2.9.2 [X], 2.10.1 [ ], 5.6.2 [ ], 5.7.1 [ ], 3.2 [ ], 7.2 [ ], other: |
| 4. | Products covered (HS or CCCN where applicable, otherwise national tariff heading. ICS numbers may be provided in addition, where applicable): Packaging materials and accessories (ICS code(s): 55.040) |
| 5. | Title, number of pages and language(s) of the notified document: Draft of the ES for "test method for determination of leaks in flexible packaging by bubble emission"; (13 page(s), in Arabic) |
| 6. | Description of content: 1.1 This draft Standard covers the determination of gross leaks in flexible packaging containing a headspace gas. Test sensitivity is limited to 1 × 10−5 atm cm3/s (1 × 10−6 Pa m3/s) or even less sensitive as indicated in a recent inter laboratory test (reported in Section 12). 1.2 Small leaks may not be detected by this procedure. Viscoelastic effects on the products, or entrapped air, become significant and prevent passage through small openings. Positive pressure inside the pouch after the vacuum is drawn may force the product to plug small leaks. The size of the leak that can be detected is dependent upon the products contained, the nature of the packaging material, and the test parameters selected. 1.3 The values stated in inch-pound units are to be regarded as standard. The values given in parentheses are mathematical conversions to SI units that are provided for information only and are not considered standard. 1.4 This standard does not purport to address all of the safety concerns, if any, associated with its use. It is the responsibility of the user of this standard to establish appropriate safety, health, and environmental practices and determine the applicability of regulatory limitations prior to use. Worth mentioning is that this draft standard is technically identical with ASTM D3078 − 2021 |
| 7. | Objective and rationale, including the nature of urgent problems where applicable: Safety and environmental requirements |
| 8. | Relevant documents: ASTM D3078 – 2021. |
| 9. | Proposed date of adoption: To be determined Proposed date of entry into force: To be determined |
| 10. | Final date for comments: 60 days from notification |
| 11. | Texts available from: National enquiry point [X] or address, telephone and fax numbers and email and website addresses, if available, of other body: Egyptian Organization for Standardization and Quality Address: 16 Tadreeb El-Modarrebeen St. Ameriya, Cairo – Egypt E-mail: eos@idsc.net.eg / eos.tbt@eos.org.eg Website: http://www.eos.org.eg |